SPECIFICATIONS FLEXGUIDE®

Product	: RENSON LOGGIA® SLIDING PANELS

Type		: FLEXGUIDE® (PA-G-0.0020)

Description

RENSON Flexguide® is a self-adjusting bottom channel that uses spring-loading to adapt to height differences (e.g. slopes, construction tolerances, thermal expansion, etc.). Thus, filling the bottom channel profile on gradients (up to 50 mm) is unnecessary. The bottom channel profile fits simply to the surface. Even on slopes, surfaces that are not entirely flat and where dimensional changes arise (e.g. as a result of temporary loading or thermal expansion), the sprung pin in the Flexguide® system maintains continuous contact with the bottom channel profile.

Standards

- Strength calculations are based on the following standards:

- ENV 1999-1-1: Design of aluminium structures

- NBN B-03-002-2: “Wind load – Dynamic effects”

- EN 1991-1-4: Wind actions

- Sliding panels equipped with Flexguide® meet all CE requirements as stated in Declaration of Performance RP/001 (based on harmonised technical specifications in accordance with EN 13659:2004).

Material

- Housing: stainless steel (SS)

- Pin: POM plastic, black

Design

- Types and applications

- Flexguide® “Plus” T-profile, 37x12,5 mm

  - Shallow (12,5 mm) “T” bottom channel profile – suitable for openings in, for example, terrace covering systems.

- Flexguide® “long” L-profile, 25 x 25 x 4 mm

  - “L” bottom channel profile, height 25 mm – suitable where wind loads are severe.

- System properties

- Flexguide® has a sprung pin that enables the system to compensate for height differences of up to 50 mm.

- Flexguide® can, with no visible fittings, be discretely integrated into the sliding panel’s vertical frame profile.

- Flexguide® is made entirely from stainless steel and is maintenance-free.

- Flexguide® has been through a development programme that included durability testing. Long life with full functionality is guaranteed.